# III B. Tech I Semester Regular Examinations -  December, 2023

APPLIED THERMODYNAMICS

(MECH)

# Time: 150 Min                                                             		                 Max. Marks:80M

# SECTION – A

Answer all Four questions				                                                 4×10M=40M

1.  a) Explain with neat sketch working of single-stage single-acting reciprocating air

compressor.  									               [2M]

b) Derive the expression of volumetric efficiency for reciprocating air compressor.          [4M]

c) A single-stage single-acting air compressor is required to deliver 14 m3 of air per minute          measured at 1.013 bar and 15°C. The delivery pressure is 7 bar and the speed 300 r.p.m.          Take the clearance volume as 5% of the swept volume with the compression and expansion

index of n = 1.3. Calculate: (i) Swept volume of the cylinder; (ii) The delivery temperature;            (iii) Indicated power. 									   [4M]

2.  a) List out the performance parameters of internal combustion engines. 			   [2M]

b) List out the difference between 2stroke engine and 4-stroke engine. 			   [4M]

c) A 4-cylinder two-stroke cycle petrol engine develops 30 kW at 2500 r.p.m. The mean          effective pressure on each piston is 8 bar and mechanical efficiency is 80%. Calculate the          diameter and stroke of each cylinder of stroke to bore ratio 1.5. Also calculate the fuel          consumption of the engine, if brake thermal efficiency is 28%. The calorific value of the          fuel is 43900 kJ/kg. 									   [4M]

Top of Form

3. a) List out the limitations of Carnot cycle. 						   [2M]

b) Draw the p-v and T-S diagrams for Rankine cycle. 					   [4M]

c) In a steam turbine steam at 20 bar, 360°C is expanded to 0.08 bar. It then enters a         condenser, where it is condensed to saturated liquid water. The pump feeds back the water          into the boiler. Assume ideal processes, find per kg of steam the net work and the cycle         efficiency. 	                                                                                                               [4M]

4.  a) Draw the p-v and T-S diagrams for re-heat Rankine cycle.  				   [2M]

b) List out the advantages of re-heat cycle. 						   [4M]

c) Steam at a pressure of 15 bar and 250°C is expanded through a turbine at first to a pressure

of 4 bar. It is then reheated at constant pressure to the initial temperature of 250°C and is finally expanded to 0.1 bar. Estimate the work done per kg of steam flowing through the turbine and amount of heat supplied during the process of reheat. Compare the work output

when the expansion is direct from 15 bar to 0.1 bar without any reheat. Assume all expansion processes to be isentropic.							   [4M]

SECTION - B

Answer all Two questions 	 					                         2×20M=40M

- a) Explain with neat sketch working of De-Laval impulse turbine. 			   [4M]

b) Contrast the working of pressure compounded over velocity compounded impulse turbine.

[8M]

c) A single stage steam turbine is supplied with steam at 5 bar, 200°C at the rate of 50 kg/min.

It expands into a condenser at a pressure of 0.2 bar. The blade speed is 400 m/s. The     nozzles are inclined at an angle of 20° to the plane of the wheel and the outlet blade angle is

30°. Neglecting friction losses, determine the power developed, blade efficiency, and stage     efficiency.  	                                                                                                               [8M]

- a) List out the components used in gas turbine cycle and mention thermodynamic process in

each of it. 										   [4M]

b) List out the advantages of gas turbines over steam turbines. 				   [8M]

c) A gas turbine plant consists of two turbines. One compressor turbine to drive compressor    and other power turbine to develop power output and both are having their own combustion chambers which are served by air directly from the compressor. Air enters the compressor at 1 bar and 288 K and is compressed to 8 bar with an isentropic efficiency of 76%. Due to heat added in the combustion chamber, the inlet temperature of gas to both turbines is 900°C. The isentropic efficiency of turbines is 86% and the mass flow rate of air at the compressor is      23 kg/s. The calorific value of fuel is 4200 kJ/kg. Calculate the output of the plant and the thermal efficiency if mechanical efficiency is 95% and generator efficiency is 96%. Take cp = 1.005 kJ/kg K and γ = 1.4 for air and c                             pg = 1.128 kJ/kg K and γ = 1.34 for gases.                                                                      [8M]