III B. Tech I Semester Regular Examinations –December, 2024

APPLIED THERMODYNAMICS

(MECH)

Time: 150 Min                                               	Max. Marks: 60M

Note: Use of Steam Table Data Book is permitted

Students are requested to collect Steam Table Data Book from invigilator

# SECTION – A

Answer all Four questions		                                                                             4×8M=32M

1. 	a)	Delineate the design, operational principles, and performance characteristics of a single-          	stage reciprocating compressor.

b)	Derive the expression of volumetric efficiency for reciprocating air compressor.

c)	Air at 101 kPa and 30°C is drawn in low pressure cylinder of a two-stage air compressor         	and is isentropically compressed to 800 kPa. The air is then cooled at constant pressure        	to 37°C in an intercooler and is then again compressed isentropically to 4 MPa in the high 	pressure cylinder, and is delivered at this pressure. Determine the power required to run the

compressor if it has to deliver 30 m3 of air per hour measured at inlet conditions.

2.	a)	Discuss the advantages and applications of Multi-stage reciprocating compression.

b)	List out the performance parameters of internal combustion engines.

c)	A four cylinder four-stroke S.I. engine has a compression ratio of 8 and bore of 100 mm,         	with stroke equal to the bore. The volumetric efficiency of each cylinder is equal to 75%.          	The engine operates at a speed of 4800 r.p.m. with an air-fuel ratio 15. Given that                  	the calorific value of fuel = 42 MJ/kg, atmospheric density = 1.12 kg/m3, mean effective          	pressure in the cylinder = 10 bar and mechanical efficiency of the engine = 80%, determine

the indicated thermal efficiency and the brake power.

3. 	a)	List out the comparison between Rankine Cycle and Carnot Cycle.

b)	Draw the p-v and T-S diagrams for Rankine cycle.

c)	In a steam turbine steam at 20 bar, 360°C is expanded to 0.08 bar. It then enters a 	condenser, where it is condensed to saturated liquid water. The pump feeds back the water 	into the boiler. Assume ideal processes, find per kg of steam the net work and the cycle 	efficiency.

4. 	a)	Discuss the supersaturated or metastable expansion of steam in a nozzle.

b)	Draw the p-v and T-S diagrams for re-heat Rankine cycle.

c) Steam at a pressure of 15 bar and 250°C is expanded through a turbine at first to a pressure         	of 4 bar. It is then reheated at constant pressure to the initial temperature of 250°C and is          	finally expanded to 0.1 bar. Estimate the work done per kg of steam flowing through the          	turbine and amount of heat supplied during the process of reheat. Compare the work output

when the expansion is direct from 15 bar to 0.1 bar without any reheat. Assume all          	expansion processes to be isentropic.

SECTION- B

Answer All Two Questions                                                                                        2x14M=28M

5. 	a)	Provide a comprehensive overview of steam turbine and its applications.

b)	Explain with neat sketch working of De-Laval impulse turbine.

c)	A single stage steam turbine is supplied with steam at 5 bar, 200°C at the rate of 50 	kg/min. It expands into a condenser at a pressure of 0.2 bar. The blade speed is 400 m/s.           	The nozzles are inclined at an angle of 20° to the plane of the wheel and the outlet blade           	angle is 30°. Neglecting friction losses, determine the power developed, blade efficiency,          	and stage efficiency.

6. 	a)	List out the components used in gas turbine cycle and mention thermodynamic process in         	each of it.

b)	Discuss the merits and demerits of gas turbines.

c)	The air enters the compressor of an open cycle constant pressure gas turbine at a pressure 	of 1 bar and temperature of 20°C. The pressure of the air after compression is 4 bar. The       	isentropic efficiencies of compressor and turbine are 80% and 85% respectively. The air-	fuel ratio used is 90: 1. If flow rate of air is 3.0 kg/s, find: (i) Power developed. (ii) 	Thermal efficiency of the cycle.