III B. Tech II Semester Regular Examinations –May, 2024

BIOMECHANICS

Time: 150 Min                                            (R&A)                                       Max. Marks:80M

# SECTION – A

Answer all four questions	                                                                              4×10M=40M

1.   a. How biomechanics is related to the area of mobile robotics

b. Illustrate how knowledge of anatomical structures influences the design of sports     equipment.

c. What is the importance of anatomical planes and discuss various anatomical movements 	described with respect to these planes.

2.   a. What do you understand the concept of complex loading subjected to any typical bone

b. Apply Mohr’s circle concept to a component subjected to simultaneous bending moment 	and twisting moment.

c. Why failure theories are used to determine the factor of safety and discuss any two 	theories of failure.

3. a. What do you mean by strain and strain rate and also write their relations with 	viscoelastic stress

b. Analyse various viscoelastic models under loading conditions. Consider factors such 	as stress relaxation and creep in the tissue.

c. Discuss the viscoelastic characteristics of tendons and ligaments during application of 	mechanical loads.

4.  a. Differentiate linear and angular kinematics in human movement analysis.

b. Apply the principles of linear kinematics to analyze the trajectory of a basketball player during a layup.

c. Analyse how angular movements of body segments contribute to the overall linear motion of the athlete.

SECTION - B

Answer all two questions 					                              2×20M=40M

5. a. What is the type of joint located in the shoulder and what are various movements of 	shoulder?

b. Determine the magnitude FM of the force exerted by the deltoid muscle to hold the arm 	at the position shown below.

c. Also determine the magnitude and direction of the reaction force at the shoulder joint    in terms of specified parameters.

O: axis of rotation

A: where the deltoid muscle is attached

B: center of gravity of the entire arm,

C: center of gravity of the dumbbell.

W : weight of the arm,

WO :weight of the dumbbell,

FM: magnitude of the tension in the deltoid muscle,

FJ:  magnitude of the joint reaction force at the shoulder.

β: angle made by resultant deltoid muscle force makes

The distances between point O and points A, B, and C are measured as a, b, and c, respectively.

6. 	a. Determine the Poisson’s ratio if material modulus of rigidity, G is 80 GPa and 	Young’s modulus is 2.25 G.

b. For a general 2D stress state system, find the principal stresses using graphical 	method.

c. Why failure theories are used in the design of machine components. Discuss any two 	theories of failure to find the factor of safety.