I B. Tech I/II Semester Supplementary Examinations – July-2025

BASICS ELECTRICAL & ELECTRONICS ENGINEERING

(CSE/CSE(CS)/CSE(AI&ML)

Time: 150 Min                                                                                             Max. Marks:60M

# SECTION – A

Answer all Four questions		                                                                     4×8M=32M

1. The circuit is as shown below

- Does Ohm’s law apply to linear elements? Justify	                                             [2M]

- When 100 volts applied between terminals Evaluate (i) Equivalent resistance across

the source and (ii) the current supplied by the source			         [3M]

- Verify Power delivered by the source is equal to the power dissipated by the resistors for the above circuit?                                                                                             [3M]

2. The primary winding of a transformer has 500 turns, and secondary turns 600 When connected across a 220V, 50cycles/sec supply

a) Describe how a transformer works based on the concept of mutual induction.     [2M]

b) Determine Maximum value of the core flux and secondary induced emf 	         [3M]	c) Evaluate the i) input power ii) output power 				         [3M]

3.   A 4-pole, three phase induction motor operates from a supply whose frequency is 60Hz.

a)  Describe the working principle of a three-phase induction motor using a rotating magnetic field								          [2M]

b)  Evaluate (i) speed of the rotating magnetic flux wave (ii) speed of the rotor when  slip

is 3%. 				                                                                      [3M]

c) If the rotor running at 1500 RPM, evaluate (i) Slip speed (ii) Slip (iii) frequency of rotor at standstill. 								         [5M]

4. Design a mobile charger, which consists of centre-taped transformer to convert the AC supply available in the electrical board of house from high voltage to the low voltage, and use voltage regulator to make constant DC output. The specifications are as follows:

a) Analyse the Applications of PN diode and Zener diode?			         [2M]

b) From Electrical board of house: Voltage: 240 Volt, AC Frequency:50 Hz              [3M]

c) To the output of Charger: Voltage: 7 Volt, DC 				         [3M]

SECTION - B

Answer all Two questions 	 				                              2×14M=28M

5. For the following circuit,

- Is the above circuit can be solved by nodal analysis? Justify?		          [4M]

- Evaluate i) current flowing through 20Ω and 30Ω resistors ii) Power supplied / absorbed by 80 V source? 							          [5M]

- Evaluate voltage drop across each resistor?                                                               [5M]

6. A capacitance of 100 µF having an internal resistance of 25Ω is connected in parallel to an inductance of 50 mh having an internal resistance of 20Ω.

a)  Is inductive reactance inversely proportional to frequency of the supply. Justify?   [4M]

b)  If this combination is excited by an AC supply of 220V, 50 Hz. Find impedance offered   by each branch, equivalent impedance, current in each branch, total Suggest an element which is to be connect in series to above combination to make maximum current supplied by source and value of maximum current also.                                                           [5M]

c)  Find Current, phase angle, power factor and P, Q and S.			          [5M]