II B. Tech II Semester Supplementary Examinations –July, 2024

COMPUTER ORGANIZATION AND ARCHITECTURE

Time: 150 Min                                               (CSE)                                     Max. Marks:80M

# SECTION – A

Answer all Four questions		                                                                    4×10M=40M

- The outputs of four registers, R0, Rl, R2, and R3, are connected through 4-to-1-line multiplexers to the inputs of a fifth register, RS. Each register is eight bits long. The required transfers are dictated by four timing variables

T0 through T3, as follows:

T0: R5 <-- R0

T1: R5 <-- Rl

T2: R5 <-- R2

T3: R5 <-- R3

The timing variables are mutually exclusive, which means that only one timing variable is equal to 1 at any given time, while the other three are equal to 0.

a) Write a short notes on Register Transfer language.                                             [3M]

b) Draw a block diagram of the hardware implementation for register transfers.      [3M]

c) Design the necessary connections from the four timing variables to the selection inputs of the multiplexers and to the load input of register R5.                                [4M]

- a) List out the various program control Instructions.                                               [3M]

b) If SP=000000 in the stack then determine the number of items present in the stack when

i) FULL=1 and EMPTY=0.

ii) FULL=0 and EMPTY=1 (Apply)                                                                 [3M]

c) Assume Stack Pointer has been initialized to 5000 and it stores the first element of the stack. Categorize the micro-operations for push and pop operations for the following data.							                      [4M]

| PC | Program (instructions) | 2000 |
| --- | --- | --- |
| AR | Address (Operands) | 3000 |
| SP | Stacks | 4000 |
|  |  | 4001 |
|  | . | . |
|  | . | . |
|  |  | 5000 |

- a) Contrast the various modes of data transfer in input-output organization.          [3M]

b) Mention the necessity of an Input-Output Interface module.                               [3M]

c) With the help of a flow chart, analyze why programmed I/O is not preferable. And also mention the remedies to be considered to overcome the problem associated with programmed I/O with the help of block diagrams.                                               [4M]

4. a) Mention the applications of logic micro-operations.                                          [4M]

b) Derive the number of logical functions that can be generated by a 2-bit binary number. 						        		                                            [2M]

c) Tabulate the obtained logical functions and mention the type of micro operation it performs.  Also Implement the hardware module of logic micro operation.      [4M]

SECTION - B

Answer all Two questions 	 					                    2×20M=40M

- a) Illustrate how PUSH and POP operations are performed in Stack.                    [5M]

b) Mention how they are executed in the case of a Memory stack organization.   [5M]

c) Evaluate the Arithmetic statement X = A + B * [ C * D + E * (F + G)]           [10M]

i) Using an accumulator type computer with one address instructions.

ii) Using a stack organized computer with zero address instructions.

6.   a) List out the differences between Von Neumann and Hardward architectures.    [5M]

b) Develop a Flowchart of a composite data path to implement the division of two   unsigned numbers. 						                                 [5M]

c) Considering an example analyze how its acts as a restoring algorithm.              [10M]