II B. Tech II Semester Regular/Supplementary Examinations –May, 2025

COMPUTER ORGANIZATION & ARCHITECTURE

(CSE/CSE(AI&ML))

Time: 150 Min 	                                                                                         Max. Marks:60M

SECTION – A

Answer all Four questions				                                                   4×8M=32M

| 1. | A typical digital computer has many registers, and paths must be provided to transfer information from one register to another. The number of wires will be excessive if separate lines are used between each register and all other registers in the system. A more efficient scheme for transferring information between registers in a multiple-register configuration is a common bus system. A bus structure consists of a set of common lines, one for each bit of a register, through which binary information is transferred one at a time. Control signals determine which register is selected by the bus during each particular register transfer. |  |
| --- | --- | --- |
| (a) | Illustrate a System bus system with tri-state buffers. | [3M] |
| (b) | Design the bus system for four registers using multiplexers and each register has four bits, numbered 0 through 3. | [3M] |
| (c) | Which scheme in the above bus system can transfer information from any register to any other? Justify. | [2M] |
| 2. |  |  |
| (a) | State the importance of floating-point representation over fixed-point representation. | [2M] |
| (b) | Convert the following number into IEEE 754 single-precision floating-point format. −1.10001111001 × 2 1001101 | [3M] |
| (c) | Convert the following numbers into IEEE 754 double-precision floating-point format. 1.1011110000101 × 2 −1010001 | [3M] |
| 3. |  |  |
| (a) | Differentiate various cache memory mapping techniques | [2M] |
| (b) | By applying the cache direct mapping knowledge, realize a map between 8K×8 Main memory with 128×8 Cache Memory. Use hexadecimal notation to represent address and data. | [3M] |
| (c) | Give the comparative analysis of the directive cache memory mapping with the two-way set associative cache memory mapping technique which maps the above 8K×8 primary memory with 128×8 cache memory. | [3M] |
| 4. |  |  |
| (a) | Write a short note on peripheral devices in input-output organization. | [2M] |
| (b) | Why does the CPU not access I/O devices, and how do you resolve this with the help of the input-output interface? | [3M] |
| (c) | A bus-organized CPU has 16 registers and 32 bits in each, with an ALU and a decoder: - Determine the size of each multiplexer and give the number of multiplexers in Bus A. - Create a control word for the system, considering that the ALU has 32 operations. | [3M] |

# SECTION - B

Answer all Two questions                                                                                     2×14M=28M

| 5. | The outputs of four registers, R0, R1, R2, and R3, are connected through 4-to-1-line multiplexers to the inputs of a fifth register, RS. Each register is eight bits long. Four timing variables dictate the required transfers T0 through T3, as follows: T0: R5 R0 T1: R5 R1 T2: R5 R2 T3: R5 R3 The timing variables are mutually exclusive, which means that only one timing variable is equal to 1 at any given time, while the other three are equal to 0. |  |
| --- | --- | --- |
| (a) | Write short notes on Register Transfer language. | [3M] |
| (b) | Analyze the above scenario and suggest a block diagram for the hardware implementation for register transfer. | [5M] |
| (c) | Design the necessary connections from the four timing variables to the selection inputs of the multiplexers and to the load input of register R5. | [6M] |
| 6. |  |  |
| (a) | How does Associative memory access differ from Main memory access? | [3M] |
| (b) | Design the hardware block for one cell of associative memory. Also, analyse how it supports memory read and write operations. | [5M] |
| (c) | Realize a logic circuit to derive the match register content shown in the figure. Argument Register; 1; 0; 1; 1; 1; 1; 1; 0; 0. Key Register; 1; 0; 1; 0; 1; 0; 1; 0; 1. Associative Memory; 1; 0; 0; 1; 1; 1; 1; 0; 0. 1; 1; 0; 0; 1; 0; 1; 0; 0. 1; 1; 1; 0; 1; 0; 1; 0; 0. 1; 1; 1; 1; 1; 0; 1; 1; 0. 0; 1; 0; 0; 0; 0; 1; 1; 0. Match Register. | [6M] |