II B. Tech II Semester Supplementary Examinations –July, 2025

COMPUTER ORGANIZATION & ARCHITECTURE

(CSE)

Time: 150 Min 	                                                                                         Max. Marks:60M

SECTION – A

Answer all Four questions				                                                    4×8M=32M

| 1. |  |  |
| --- | --- | --- |
| (a) | How does a common bus system improve the performance of a computer system? | [2M] |
| (b) | Design a common bus with multiplexers for a 4-bit CPU with 4 registers. | [3M] |
| (c) | Realize the common bus mentioned in 1.b) with Decoders and tri-state buffers. | [3M] |
| 2. | The content of the PC in the basic computer is 03AF H. The content of AC is 7EC3 H. The content of the memory at address 03AF H is 932E H. The content of memory at address 032E H is 09AC H. The content of the memory at address 09AC H is 8B9F H. Assume the opcode 001 in IR specifies the ADD operation. |  |
| (a) | What is the instruction that will be fetched and executed next? | [2M] |
| (b) | Implement the binary operation that will be performed in the AC when the instruction is executed. | [3M] |
| (c) | Examine the contents of the PC, AR, DR, AC, and IR registers in hexadecimal at the end of the instruction cycle. | [3M] |
| 3. |  |  |
| (a) | Give the reasons why we are using content-addressable memory. | [2M] |
| (b) | Derive the logical expression to find the match logic for a 3×4 associative memory. | [3M] |
| (c) | Realize the hardware block for implementing the above 3×4 associative memory. | [3M] |
| 4. |  |  |
| (a) | Differentiate Synchronous and Asynchronous transmissions. | [2M] |
| (b) | Illustrate the Mechanisms of Asynchronous Data transfer input-output organization. | [3M] |
| (c) | The system uses an asynchronous serial communication protocol with a baud rate of 19200. The data format is 1-16-P-2 (1 start bit,16 data bits, 1 parity bit, 2 stop bits). (i) Calculate the total number of bits transmitted per character. (ii) Compute the time required to transmit one character. (iii) How long will it take to transmit a 3MB file? | [3M] |

# SECTION - B

Answer all Two questions                                                                                   2×14M=28M

| 5. |  |  |
| --- | --- | --- |
| (a) | Differentiate Von Neumann and Harvard computer architectures. | [4M] |
| (b) | Design an algorithm to multiply two unsigned hexadecimal numbers. Justify it by choosing the numbers between +2 and +8. | [5M] |
| (c) | Design an ALU, for a 4-bit hypothetical CPU that performs the following operations. - Addition with carry - Ex-OR - Logical Left shift - Right shift of one bit with sign extension. | [5M] |
| 6. |  |  |
| (a) | Describe the memory hierarchy by considering various memory characteristics | [4M] |
| (b) | Consider a CPU that consists of 20 address lines. Interface the CPU with two chips of 32K × 8 PROM and four chips of 32K × 8 RAM as per the following map ROM 1 and 2: F0000H – FFFFFH, RAM 1 and 2: 60000H-6FFFFH, RAM 3 and 4: A0000H – AFFFFH. Suggest suitable address decoder circuits and memory map tables for the implementation of this memory system. | [5M] |
| (c) | Design a CPU-Main memory interface that connects the processor with four128×8 RAMs and one 512×8 ROMs. Analyse it with suitable memory map tables. | [5M] |