II B. Tech I Semester Supplementary Examinations – December 2024

DIGITAL LOGIC & COMPUTER ORGANIZATION

Time: 150 Min                                         (CSE(CS))                                        Max. Marks:60M

SECTION – A

Answer all Four questions                                                                                       4×8M=32M

- A sequential circuit has two D flip-flops A and B, two inputs x and y, and one output z. The flip-flop input equations and the circuit output are as follows:

DA= xʹy + xA

DB= xʹB + xA

z = B

- Draw the logic diagram of the circuit

- Tabulate the state table.

2.    a)   List and explain Arithmetic, Logic and Shift Micro operations.

b)  A computer uses a memory unit with 256K words of 32 bits each. A binary 	instruction code is stored in one word of memory. The instruction has four parts: an 	indirect bit. an operation code, a register code part to specify one of 64 registers, and 	an address part.

- How many bits are there in the operation code, the register code part, and the address part?

- Draw the instruction word format and indicate the number of bits in each part.

- How many bits are there in the data and address inputs of the memory?

3.   a) What is the difference between isolated I/O and memory-mapped I/O? What are the  advantages and disadvantages of each?

b) Why are the read and write control lines in a DMA controller bidirectional? Under what condition and for what purpose are they used as inputs? Under what condition and for what purpose are they used as outputs?

4.  a) Convert the following numbers with the indicated bases to decimal:

(i) (4310)5 	(ii) (198)12	(iii) (435)8	 (iv) (345)6

b) Design a combinational circuit with three inputs x, y, z and three outputs A, B, C. When the binary input is 0, 1, 2, or 3, the binary output is one greater than the input. When the binary input is 4, 5, 6, or 7, the binary output is one less than the input.

SECTION-B

Answer all Two questions						                 2×14M=28M

5. Given the Boolean function

F = xyʹz + xʹyʹz + xyz

- List the truth table of the function.

- Draw the logic diagram using the original Boolean expression.

c)  Simplify the algebraic expression using Boolean algebra.

d) List the truth table of the function from the simplified expression and show that it  	is the same as the truth table in part (a).

e) Draw the logic diagram from the simplified expression and compare the total number of gates with the diagram of part (b).

6. The outputs of four registers, RO, Rl, R2 and R3, are connected through 4-to-1 line multiplexers to the inputs of a fifth register, R5. Each register is eight bits long. The required transfers are dictated by four timing variables

To through T3, as follows:

T0:	 R5 ← R0

T1:	 R5 ← R1

T2:	 R5 ← R2

T3:	 R5 ← R3

The timing variables are mutually exclusive, which means that only one variable is equal to 1 at any given time, while the other three are equal to 0. Draw a block diagram showing the hardware implementation of the register transfers. Include the connections necessary from the four timing variables to the selection inputs of the multiplexers and to the load input of register R5.