II B. Tech I Semester Regular Examinations – December, 2023

DIGITAL LOGIC & COMPUTER ORGANIZATION

Time: 150 Min                       (cse(Cs)/cse(DS)/cse(aiml))              Max. Marks:80M

# SECTION – A

Answer all Four questions		                                                                    4×10M=40M

- a) Convert the following numbers to decimal system 			          [3M]

i. (1001001.011)2              ii. (1342)8           iii. (198)12

b) Perform the subtraction with the following binary numbers using 1’s complement

i. (11010-1101)2      ii. (10010-10011)2                iii. (11010-10000)2                                [3M]

c) Implement the F(a,b,c)=∑(1,2,3,5,7) using different gates.                                 [4M]

- a) Define the universal gates. Implement basic gates using the two universal gates. [3M]

b) Design the full added and explain with the truth table.                                        [3M]

c) F=A(B+CD) + BC’ with NAND gates.                                                                [4M]

- a) An 8-bit register contains the binary value 10011100. What is the register value after

an arithmetic shift right and shift left. and state whether there is an overflow ?      [4M]

b) Explain the various logical micro-operations with the notations.                                  [3M]

c) Starting from an initial value of R =11011101, determine the sequence of binary

values in R after a logical shift-left, followed by a circular shift-right, followed by a

logical shift-right and a circular shift-left.					          [4M]

- a) What is the difference between a direct and an indirect address instruction?      [3M]

b) Illustrate how many references to memory are needed for such type of instruction to bring an operand into a processor register?                                                         [3M]

c) Consider the instruction formats of the basic computer, for each of the following 16-bit instructions, give the equivalent four-digit hexadecimal code and explain in your own words what it is that the instruction is going to perform.                           [4M]

i)   0001 0000 0010 0100

ii)  1011 0001 0010 0100

iii) 0111 0000 0010 0000

SECTION - B

Answer all Two questions 	 					                    2×20M=40M

- a) Explain the mapping rules of a K-map with example. 			          [5M]

b) Simplify the Boolean function F(x, y, z)=∑(0,2,4,5,6)    			          [5M]

c) Simplify the Boolean function                                          			        [10M]

i)  f (w, x, y, z)= ∑(0,1,2,4,5,6,8,9,12,13,14)

ii) f (a, b, c, d) = a’b’c’+b’cd’+a’bcd’+ab’c’

- a) Explain the instruction codes? Explain the different instruction code formats. [6M]

b) Specify an instruction format for a computer that performs the following operation:

A M[address]+R

where R can be any one of eight possible registers in the processor.                [6M]

c)  Design an arithmetic circuit with two selection variables, s1 and s0, that generates

the following arithmetic operations. Draw the logic diagram of one typical stage.

[8M]

| S1 | S0 | Cin=0 | Cin=1 |
| --- | --- | --- | --- |
| 0 | 0 | F=A+B | F=A+B+1 |
| 0 | 1 | F=A | F=A+1 |
| 1 | 0 | F=B’ | F=B’+1 |
| 1 | 1 | F=A+B’ | F=A+B’+1 |