I B.Tech II Semester Supplementary Examinations, November, 2023

SEMICONDUCTOR PHYSICS AND DEVICES

(ECE)

# Time: 150 Min                                                     	                                   Max. Marks: 80M

# SECTION – A

Answer all Four questions					                                     4×10M=40M

1.a) Compare the V-I characteristics of a germanium and a silicon diode graphically with respect to Temperature variations

b) Analyze the forward voltage drop across the diode whose characteristics appear in Fig. 1 at temperatures of -75°C, 25°C, 100°C, and 200°C and a current of 10 mA. For each temperature, determine the level of saturation current. Compare the extremes of each and comment on the ratio of the two.

Fig. 1

2. Voltage regulator is shown in fig 2where Vs is an unregulated voltage that varies between 6V and 7V while the Zener diode voltage is VZ = 5 V. The load resistor RL can have a value from 100 Ω to ∞ (i.e., open circuit). Also you can take IZ(min) ∼= 0 A.

a) Identify the maximum value of R1 so that the load voltage VL would be still kept constant at 5V for all values of RL and Vs.

b) Provide a symbolic expression for the maximum power dissipated by the Zener diode.

c) Identify the minimum value of R1 so that the power dissipated by the Zener diode does never exceeds 1W for all values of RL and Vs

Fig 2

3. Design a DC Power Supply using half-wave and bridge for the following specifications.

- Power source:220V, 60Hz, single phase AC.

- Output current: 2000mA Max

- Output voltage: 5 VDC

- Ripple Voltage: less than 0.5V for all currents

4.Calculate the voltage gain (AV=VLVi) for the network of Fig.3 if Vi=500 mV and R=1 kΩ. (The other circuit values remain the same.)

Fig. 3

# SECTION – B

Answer all Two questions					2×20M=40M

5.a) Draw the experimental setup of CE amplifier to obtain the input and output characteristics

(b) Using the characteristics of Fig. 4, determine hfe at IC = 6 mA and VCE = 5 V.

(c) Using the characteristics of Fig. 5, determine hoe at IC= 6 mA and VCE= 5 V.

(d) Using the characteristics of Fig. 6, determine hie at IB= 20 µA and VCE =20 V.

(e)Using the characteristics of Fig.7, determine hre at IB =20µ A.

(f)Using the characteristics of Figs. 4 and 5, analyze the approximate CE hybrid equivalent model at IB= 25µA and VCE =12.5 V

Fig.4                                                                                  Fig.5

Fig. 6                                                           Fig. 7

- Using the characteristics of Fig. 8, design a voltage-divider circuit with a saturation current level of 10 mA and a Q-point one-half the distance between cutoff and saturation. The available supply is 28 V, and VE is to be one-fifth of VCC. The condition established by Equation:   βRE≥10R2 should also be met to provide a high stability factor. Use standard values.

Fig. 8