I B Tech I/II Semester Supplementary Examinations – January, 2026

SEMICONDUCTOR PHYSICS & ELECTROMAGNETICS

(CSE/EEE/IT)

Time: 150 Min 	                                                                                    Max. Marks:60M

SECTION – A

Answer all Four questions				                                              4X8M=32M

| 1. |  |  |
| --- | --- | --- |
|  | Differentiate stimulated emission from spontaneous emission. | [3M] |
|  | What are the characteristic features of laser light that differs from normal light? | [3M] |
|  | Determine the band gap of a semiconducting laser emitting wavelength of 632 nm. | [2M] |
| 2. |  |  |
|  | State Biot savart’s law. | [2M] |
|  | Apply Biot savart's law to determine the magnetic field of a circular ring of radius ‘a’ at a distance ‘x’ on its axis | [3M] |
|  | Estimate the magnetic field using Biot-Savart’s law on the axis of a circular coil of radius 0.3m, with 100 turns and carrying a steady current 0.6 Amp at a point P which is at a distance 0.15m from the centre of the coil. (Given, µ0 = 4πx10-7 H.m-1). | [3M] |
| 3. |  |  |
|  | What are de Broglie waves and write the properties. | [3M] |
|  | Derive an expression for the de Broglie wavelength of a charged particle of charge ‘q’ moving under potential difference ‘V’ | [3M] |
|  | Find the wavelength of an electron accelerating under the potential difference of 30KV. | [2M] |
| 4. |  |  |
|  | Differentiate the direct and indirect bandgap semiconductors with examples. | [3M] |
|  | Are indirect bandgap semiconductors useful to fabricate LED’s? Give your reasons with examples. | [3M] |
|  | Find the wavelength of light emitting from a LED fabricated using a semiconductor material of bandgap 3 eV. | [2M] |

# SECTION - B

Answer all Two questions                                                                                    2X14M=28M

| 5. |  |  |
| --- | --- | --- |
|  | Apply Schrodinger wave equation to derive an expression for energy eigen value of a particle struck in a one-dimensional infinite potential well of width ‘a’. | [4M] |
|  | An electron is confined within an infinite potential well of width a = 10 nm. Calculate the energy of emitted photon when an electron jumps from the third excited state to ground state. | [4M] |
|  | Derive coulombs law from Gauss law. | [3M] |
|  | Apply gauss law to find the electric filed of a spherical shell of radius ‘a’, in the following cases - inside (ii) on the shell (iii) outside the shell | [3M] |
| 6. |  |  |
|  | What is Hall effect and derive an expression for hall voltage. | [4M] |
|  | Hall coefficient of a p-type silicon found to be 3 ×10–3 m3C–1. The resistivity of the specimen is 9 × 10–3 Ωm. Find the mobility and density of the charge carriers. | [4M] |
|  | Describe the construction and working of solar cell. | [3M] |
|  | A solar cell has an input power of 200 W/m2 from sunlight and generates an output power of 40 W. Calculate the efficiency of this solar cell. | [3M] |