# II B.Tech I Semester Supplementary Examinations – September, 2023

# Signals and Systems

# Time: 150 Min                                              (ece)	            	            Max. Marks:80M

# SECTION – A

Answer all Four questions					                                4×10M=40M

- Given x1t=cost, x2t=sinπt and x3t=x1t+x2t.

- Determine the fundamental periods T1 and T2of signals x1(t) and x2(t) respectively.

- Show that x3t is not periodic, which requires T3=k1T1=k22 for some integers k1 and k2.

- Determine the powers Px1, Px2 and Px3 of signals x1t, x2t, and x3t

- An analog LTI system with impulse response function ht=ut+3-u(t-3) is presented with an input xt=t[ut-u(t-3)].

- Determine and plot the system output yt=xt*h(t)

- Is the system BIBO-stable? Justify your answer.

- Is the system time invariant? Why?

- Is the system causal? Justify.

- Consider the circuit shown in Fig. 1.

- Determine the standard form constant coefficient differential equation description of this circuit.

- Letting R=C=1, determine the response y(t) to input xt=3e-tu(t) and initial capacitor voltage of Vc0-=5.

Fig. 1

- A periodic signal x(t) is expressed by the following Fourier series:

xt=3cost+sin(t-π6)-2cos(t-π3)

- Sketch the amplitude and phase spectra for the trigonometry series.

- By inspection of spectra in part (a), sketch the exponential Fourier series spectra.

- By inspection of spectra in part (b), write the exponential Fourier series for x(t).

SECTION - B

Answer all Two questions 	 					                    2×20M=40M

- A complex signal x(t) has a spectrum given as

Xω=ω     -20π≤ω≤20π0           otherwise

Let x(t) be sampled at rate fs=24 Hz to produce signal x(t) with spectrum X(ω).

- Determine and sketch X(ω)

- Has aliasing occurred in sampling x(t) to produce x(t)? Explain.

- Can x(t) be exactly recovered from x(t)? Explain.

- A periodic signal xt=1+2cos5πt+3sin8πt is applied to an LTI causal system with impulse response ht=8sinc4tcos(2πt) to produce output yt=xt*h(t).

- Determine the fundamental radian frequency ω0 of x(t).

- Determine the Fourier transform of x(t).

- Sketch the system’s magnitude response |Hω| over -10π≤ω≤10π.

- Is the system h(t) distortionless? Explain.

- Determine the response of the system y(t).